# Petrocity Bonje POC Findings: Octane Orbit and ROHE FDC Integration

Oct 29, 2026 · @Jeremy

## Result

Test site: Petrocity Bonje, 23 Sep to 27 Oct 2026 (35 days), diesel and petrol. Octane matched the FDC exactly on 31 of 35 days. Where they differ, Octane is missing transactions the FDC recorded. It never records different amounts.

| Fuel | FDC (L) | Octane (L) | Octane captured |
| --- | --- | --- | --- |
| Diesel | 655,415.9 | 637,281.8 | 97.2% |
| Petrol | 33,597.4 | 32,990.8 | 98.2% |

Excluding the first day (23 Sep), Octane captured 99.7% of diesel and 99.6% of petrol.

## Days that did not match

| Date / window | Fuel | FDC (L) | Octane (L) | FDC minus Octane (L) | Note |
| --- | --- | --- | --- | --- | --- |
| 23 Sep, full day | Diesel | 28,151.68 | 11,790.08 | 16,361.60 | Start-up day; Octane saw about 42% of the day while data was still arriving through the FDC's upload queue (to be confirmed) |
| 23 Sep, full day | Petrol | 1,005.31 | 522.94 | 482.37 | Same cause |
| 30 Sep 16:45 to 1 Oct 07:30 | Diesel | 25,816.62 | 24,044.09 | 1,772.53 | Under investigation; missing transactions to be added back (see How to fix it) |
| 30 Sep 16:45 to 1 Oct 07:30 | Petrol | 1,522.54 | 1,403.82 | 118.72 | Same window |
| 3 Oct | Petrol | 947.08 | 941.08 | 6.00 | Single small difference |
| 30 Sep 07:30 to 16:45 | Petrol | 517.88 | 518.36 | -0.48 | Octane marginally higher; rounding level |

## Why the differences happen

The differences are transactions that had not reached Octane yet. The FDC sends its data slowly, about one batch every 3 minutes, so transactions can queue for hours when the station is busy.

## How to fix it

- ROHE: increase the FDC's upload speed, or allow larger batches.

- Octane: ask ROHE to export the FDC's transaction record for 30 Sep to 1 Oct, and load those transactions into Octane exactly as the FDC recorded them, with no edits. Then repeat the comparison and report the result as it comes out. The figures in the table above stay in the report as the original results.

- Both: confirm the cause of the 23 Sep and 30 Sep to 1 Oct differences.

Once the upload speed is fixed and the gaps are filled, Octane should match the FDC 100% every day.