# Petrocity Bonje POC Findings: Octane Orbit and ROHE FDC Integration

Oct 29, 2026 · @Jeremy

## Result

Test site: Petrocity Bonje, 23 Sep to 27 Oct 2026 (35 days), diesel and petrol. Octane matched the FDC exactly on 31 of 35 days. Where they differ, Octane is missing transactions the FDC recorded. It never records different amounts.

| Fuel | FDC (L) | Octane (L) | Octane captured |
| --- | --- | --- | --- |
| Diesel | 655,415.9 | 637,281.8 | 97.2% |
| Petrol | 33,597.4 | 32,990.8 | 98.2% |

Excluding the first day (23 Sep), Octane captured 99.7% of diesel and 99.6% of petrol.

## Days that did not match

| Date / window | Fuel | FDC (L) | Octane (L) | FDC minus Octane (L) | Note |
| --- | --- | --- | --- | --- | --- |
| 23 Sep, full day | Diesel | 28,151.68 | 11,790.08 | 16,361.60 | POC start-up day: set-up was still being completed after the kick-off meeting. The rest of the day's data is still on the FDC and will reach Octane once the upload speed is fixed |
| 23 Sep, full day | Petrol | 1,005.31 | 522.94 | 482.37 | Same cause |
| 30 Sep 16:45 to 1 Oct 07:30 | Diesel | 25,816.62 | 24,044.09 | 1,772.53 | Under investigation; missing transactions to be added back (see How to fix it) |
| 30 Sep 16:45 to 1 Oct 07:30 | Petrol | 1,522.54 | 1,403.82 | 118.72 | Same window |
| 3 Oct | Petrol | 947.08 | 941.08 | 6.00 | Single small difference |
| 30 Sep 07:30 to 16:45 | Petrol | 517.88 | 518.36 | -0.48 | Octane marginally higher; rounding level |

## Why the differences happen

The differences are transactions that had not reached Octane yet. The FDC sends its data slowly, about one batch every 3 minutes, so transactions can queue for hours when the station is busy.

## How to fix it

- ROHE: increase the FDC's upload speed, or allow larger batches.

- Octane: once the backlog is cleared and the upload speed is increased, the transactions still stored on the FDC will reach Octane automatically, and Octane will reconcile with them.

- Both: confirm the cause of the 23 Sep and 30 Sep to 1 Oct differences.

Once the upload speed is fixed and the backlog is cleared, Octane should match the FDC 100% every day.